/****************************************************************************

Module

LED_SM.c

Revision

1.0.1

Description

This the service that implements a State Machine for the LEDs that will

display the passage of time while a user interacts with the ART

Notes

****************************************************************************/

/*----------------------------- Include Files -----------------------------*/

/* include header files for the framework and this service

*/

#include "ES_Configure.h"

#include "ES_Framework.h"

#include "ES_DeferRecall.h"

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "inc/hw_gpio.h"

#include "inc/hw_sysctl.h"

#include "driverlib/sysctl.h"

#include "driverlib/pin_map.h"	// Define PART_TM4C123GH6PM in project

#include "driverlib/gpio.h"

#include "ES_ShortTimer.h"

#include "Sensors.h"

#include "ADmulti.h"

#include <stdlib.h>

#include <stdint.h>

#include "MotorControl.h"

/*----------------------------- Module Defines ----------------------------*/

// these times assume a 1.000mS/tick timing

#define ONE_SEC 976

#define HALF_SEC (ONE_SEC/2)

#define THREE_SEC (ONE_SEC*3)

#define FIVE_SEC (ONE_SEC*5)

#define THIRTY_SEC (ONE_SEC*30)

#define FORTY_FIVE_SEC (ONE_SEC*45)

#define SAMPLE_TIME (ONE_SEC/10)

#define ALL_ON 0x7FFF

#define ALL_BITS (0xFF<<2)

/*---------------------------- Module Functions ---------------------------*/

/* prototypes for private functions for this service.They should be functions

relevant to the behavior of this service

*/

static void CheckSensors(void);

/*---------------------------- Module Variables ---------------------------*/

// with the introduction of Gen2, we need a module level Priority variable

static uint8_t MyPriority;

static uint8_t CurrentState;

/*------------------------------ Module Code ------------------------------*/

/****************************************************************************

Function

InitializeSensors

Parameters

uint8_t : the priorty of this service

Returns

bool, false if error in initialization, true otherwise

Description

Notes

****************************************************************************/

bool InitializeSensors ( uint8_t Priority )

{

ES_Event ThisEvent;

MyPriority = Priority;

/********************************************

in here you write your initialization code

*******************************************/

// Initialize port lines to receive analog inputs from SHARP Sensors

ADC_MultiInit(2);

//Set current state as standBy

CurrentState = standBy;

// post the initial transition event

ThisEvent.EventType = ES_INIT;

if (ES_PostToService( MyPriority, ThisEvent) == true)

{

return true;

}else

{

return false;

}

}

/****************************************************************************

Function

PostSensors

Parameters

EF_Event ThisEvent ,the event to post to the queue

Returns

bool false if the Enqueue operation failed, true otherwise

Description

Posts an event to this state machine's queue

Notes

****************************************************************************/

bool PostSensors( ES_Event ThisEvent )

{

return ES_PostToService( MyPriority, ThisEvent);

}

/****************************************************************************

Function

RunSensors

Parameters

ES_Event : the event to process

Returns

ES_Event, ES_NO_EVENT if no error ES_ERROR otherwise

Description

add your description here

Notes

****************************************************************************/

ES_Event RunSensors( ES_Event ThisEvent )

{

ES_Event ReturnEvent;

ReturnEvent.EventType = ES_NO_EVENT; // assume no errors

switch(CurrentState){

case standBy :

//printf("StandBy state\n\r");

// if ThisEvent is StartupTimerExpire

if((ThisEvent.EventType == ES_TIMEOUT) && (ThisEvent.EventParam == STARTUP_TIMER)){

// Set CurrentState to Sampling

CurrentState = Sampling;

// Reset and start SampleTimer

ES_Timer_InitTimer(SAMPLE_TIMER,SAMPLE_TIME);

ES_Timer_InitTimer(THIRTY_SECOND_TIMER, THIRTY_SEC);

}

break;

case Sampling :

// if ThisEvent is SampleTimerExpire

if((ThisEvent.EventType == ES_TIMEOUT) && (ThisEvent.EventParam == SAMPLE_TIMER)){

// Call CheckSensors()

CheckSensors();

// Reset and start SampleTiemr

ES_Timer_InitTimer(SAMPLE_TIMER,SAMPLE_TIME);

}

// if ThisEvent is 30SecondTimerExpire or 45SecondTimerExpire

if((ThisEvent.EventType == ES_TIMEOUT) && (ThisEvent.EventParam == THIRTY_SECOND_TIMER)){

//printf("30 second timer expire");

// Set CurrentState to be Standby

CurrentState = standBy;

}

if((ThisEvent.EventType == ES_TIMEOUT) && (ThisEvent.EventParam == FORTY_FIVE_SECOND_TIMER)){

//printf("30 second timer expire");

// Set CurrentState to be Standby

CurrentState = Ending;

}

break;

case Ending :

if((ThisEvent.EventType == ES_TIMEOUT) && (ThisEvent.EventParam == FANFARE_TIMER)){

//printf("30 second timer expire");

// Set CurrentState to be Standby

CurrentState = standBy;

}

}

return ReturnEvent;

}

/***************************************************************************

private functions

***************************************************************************/

static void CheckSensors(void)

{

// Get the current analog inputs from the 2 sensors and save as input1, input2

static uint32_t results[2];

ADC_MultiRead(results);

ES_Event SensorEvent1;

ES_Event SensorEvent2;

// Convert input1, input2 to float and multiply by 3.3/4095 to scale to voltage.

double input1 = (double)results[0]*3.3/4095;

double input2 = (double)results[1]*3.3/4095;

static uint8_t output1;

static uint8_t output2;

// If input1>0.4

if (input1>0.4){

// Set output1 = 99(1-(11.25/(input1 ñ 0.0625))/27) as uint8_t

output1 = (uint8_t)(99*(1-(11.25/(input1 - 0.0625))/27))/2;

// PostEvent MotorPostSpeed1 to Sensors module with parameter of output1

SensorEvent1.EventType = MotorPostSpeed1;

SensorEvent1.EventParam = output1;

ES_PostList03(SensorEvent1);

//printf("30 second timer init");

ES_Timer_InitTimer(THIRTY_SECOND_TIMER, THIRTY_SEC);

}

// If input2>0.4

if (input2>0.4){

// Set output2 = 99(1-(11.25/(input2 ñ 0.0625))/27) as uint8_t

output2 = 99*(1-(11.25/(input2-0.0625))/27);

// PostEvent MotorPostSpeed1 to Sensors module with parameter of output1

SensorEvent2.EventType = MotorPostSpeed2;

SensorEvent2.EventParam = output2;

ES_PostList03(SensorEvent2);

ES_Timer_InitTimer(THIRTY_SECOND_TIMER, THIRTY_SEC);

}

printf("Output1 - %i, output2 - %i\r",output1,output2);

}

/*------------------------------- Footnotes -------------------------------*/

/*------------------------------ End of file ------------------------------*/